Simple routine lift plan

Two–three pages for a known load, one crane, clear path. Not a critical-lift study and not the 13-page engineering pack.

Stop and use a signed Appointed Person plan if: utilisation ≥ 75%, two cranes, people in a basket, power lines / live plant, unknown weight, blind landing, or the path is tight. Write N/A where a field does not apply. Silent blanks are not allowed.

1. Job

| Site / area | ________________________ | Date / shift | ________________ |
| --- | --- | --- | --- |
| Plan no. | JMR-SRL-______ | Permit no. | ________________ |
| What is being lifted | ________________________ | From → to | ________________ |
| Lift type | ☐ Yard / site ☐ Deck ☐ Truck / trailer ☐ Other | Weather limit | Wind ____ Visibility ____ |
| Person in charge | ________________________ | Revision | Rev 0 |

2. Crane and load

| Crane make / ID | ________________________ | Chart on crane | ☐ Yes |
| --- | --- | --- | --- |
| SWL at this radius | ________ t at ________ m | Utilisation | ________ % |
| Load description | ________________________ | Verified weight | ________ t / kg |
| How verified | ☐ Weigh ☐ Manifest ☐ Calc | Hook + slings + shackles | ________ kg |
| Total lifted load | ________ t | Last exam / cert date | ________________ |

3. Rigging

| Sling type / legs | ________ legs: ____ | Sling WLL each | ________ t |
| --- | --- | --- | --- |
| Shackle WLL | ________ t | Min. sling angle | ________ ° |
| Hitch | ☐ Vertical ☐ Choker ☐ Basket ☐ Other | Taglines | ☐ Two ☐ One ☐ N/A |
| Gear inspected | ☐ Pass ☐ Rejected | Colour / cert in date | ☐ Yes |

4. People and comms

| Crane operator | ________________________ | Competence checked | ☐ Yes |
| --- | --- | --- | --- |
| Banksman | ________________________ | Riggers | ________________ |
| Comms | ☐ Radio ch. ____ ☐ Hand signals ☐ Both | Tested | ☐ Before first lift |

5. Sequence — tick when briefed

| 01 | ☐ Toolbox talk. This sheet read. Roles agreed. |
| --- | --- |
| 02 | ☐ Crane checks. Chart on the machine. Radio / signals tested. |
| 03 | ☐ Landing area barred. Path, overhead and live plant checked. |
| 04 | ☐ Gear inspected. Loose items off the load. |
| 05 | ☐ Hook central. Take weight slowly. Stop. Confirm stable. |
| 06 | ☐ Hoist clear. Taglines on. Nobody under the load. |
| 07 | ☐ Land under banksman. Unhook. Boom to rest. Housekeeping. |
| 08 | ☐ If the job changes — stop, rewrite this sheet, brief again. |

6. Hazards for this lift

| Situation | Hazard | Control |
| --- | --- | --- |
| Hook-on / take weight | Pinch, unstable load | Central hook. Hands clear. Taglines on. |
| Hoist / slew / travel | Clash, overload, dropped load | Chart check. Barriers. No one under load. |
| Landing | Crush, wrong spot | Area clear. Banksman in view or on radio. |
| Other (write) | ____________________ | ____________________ |

7. Sign-off — this lift, this shift

| Role | Name | Sign | Time |
| --- | --- | --- | --- |
| Person in charge |  |  |  |
| Crane operator |  |  |  |
| Banksman |  |  |  |

Disclaimer. Blank JMR template. Completing it does not authorise the lift until a competent person has filled and signed it for the actual load, crane, radius and weather. Not a substitute for a critical-lift engineering study. jmrlifting.com